The array for investigations of EAS neutron component

D.M. Gromushkin1, N.V. Ampilogov1, N.S. Barbashina1, F.A. Bogdanov1, 
R.P. Kokoulin1, V.V. Ovchinnikov1, A.A. Petrukhin1, I.A. Shulzhenko1, 
Yu.V. Stenkin1,2, I.I. Yashin1, K.O. Yurin1

1 National Research Nuclear University MEPhI (Moscow Engineering Physics Institute), Moscow 115409, Russia

2 Institute for Nuclear Research of RAS, Moscow 117312, Russia

E-mail: DMGromushkin@mephi.ru

Neutron component of EAS has not been practically studied for many years. Creation  of the PRISMA project prototypes PRISMA-32 and PRISMA-YBJ demonstrated the effectiveness of such investigations. It is important to underline that recording of  EAS neutron component is carried out over the whole array area. Development of a new scintillator array (URAN) in MEPhI will continue these studies. During the construction of the URAN array, a modernized design of en-detectors based on the scintillator with natural boron and equipped with new electronics was developed. Using of new electronics with a switchable sampling rate allows one to reach EAS fast timing accuracy up to 10 ns. The en-detectors of URAN are located on the roofs of the buildings. The sensitive area of each detector is 0.36 m2, while the array's starting area is ~103 m2. Total number of the initial array detectors is equal to 72. The are subdivided into 6 independent clusters of 12 en-detectors each, connected with the central station of data acquisition.